Anbindung einer Modbus Relaiskarte an Home Assistent über USB Modbus Master

Anleitung Quelle: Quelle: https://community.home-assistant.io/t/modbus-rtu-1-2-4-8-channel-relay-output-board/260535

USB Modbus Adapter  „USB zu RS485 Konverter Adapter CH340“  link

Relaiskarte Modbus-Rtu 4 Channel 12V Relay Module Switch Modbus RTU link

USB Modbus Adapter mit Raspberry Pi angeschlossen. (Linux Befehl dmesg um festzustellen welcher ttyUSB Port), Relaiskarte mit 12V Steckernetzteil versorgt

In Home Assistant muss nichts getan werden außer das configuration.yaml zu editieren. 
Die Entitäten sind verfügbar und „erscheinen‘“ als Switch im Dashboard Übersicht.

Konfiguration in Home Assistent  (configuration.yaml)

modbus:

- name: ModBusRelaisBoard

message_wait_milliseconds: 250

delay: 1

retries: 3

retry_on_empty: true

close_comm_on_error: false

type: serial

baudrate: 9600

bytesize: 8

method: rtu

parity: N

port: /dev/ttyUSB0

stopbits: 1

switches:

- name: SW001

slave: 1

address: 0

write_type: coil

- name: SW002

slave: 1

address: 1

write_type: coil

- name: SW003

slave: 1

address: 2

write_type: coil

- name: SW004

slave: 1

address: 3

write_type: coil

Dashboard (Entity: switch.sw001, ..)